Upper Yangtze River Scientific Data Center

Meteorological Observation Data of Citrus Research Institute of Jinfo National Station (2021)

1、Description

This data is from the observation data of the automatic weather station at the orchard observation site of the National Field Scientific Observation and Research Station of Karst Ecosystem in Jinfo Mountain, Chongqing (E106.3817569 °; N29.76232 °, 231m above sea level) from January 1 to December 31, 2021. The air temperature and relative humidity sensors of the automatic weather station in Hutou Village are respectively set up at 3m and 9m, facing due north; The barometer is installed at 10m; The tipping bucket rain gauge is installed at 10m; The wind speed and direction sensors are respectively set at 3m and 9m; The four component radiometer is installed at 5m, facing south; Photosynthetically active radiation sensor is installed at 5m and 10m, facing south; The soil temperature probes are buried 10cm, 20cm, 40cm, 60cm, 80cm, 120cm and 160cm underground, and are located in the south of the meteorological tower 2m away; The soil moisture probe shall be buried at 2cm, 5cm, 10cm, 20cm, 40cm, 60cm and 80cm underground, in the south of the meteorological tower 2m away; The soil heat flux plates (3 pieces) are successively buried at 5cm, 10cm and 20cm below the ground, just south of the meteorological tower 2m away; The average soil temperature probes (3) are buried at 5cm, 10cm and 20cm underground, and are located in the south of the meteorological tower 2m away. 1440 groups of data are observed every day (every 10min). If data is missing, it is indicated by NAN.

2、Keywords

Theme：Soil moisture dataset,Winds,Humidity/Dryness,wind speed
Discipline：Atmosphere
Places：Jinfo Mountain National Station Citrus Institute
Time：2021

3、Data details

1.Scale：None

2.Projection：

3.Filesize：17.8MB

4.Data format：None

4、Space scope

| - | north：29.76232 | - |
| --- | --- | --- |
| west：106.3817569 | - | east：106.3817569 |
| - | south：29.76232 | - |

5、Time frame:2020-12-31 16:00:00+00:00--2021-12-30 16:00:00+00:00

6、Reference method

References to data:

KONG   Debing . Meteorological Observation Data of Citrus Research Institute of Jinfo National Station (2021). Upper Yangtze River Scientific Data Center, 2022

References to articles:

7、Supporting project information

8、Data resource provider

name: KONG   Debing 
unit: Southwest University
email: kongdebing@swu.edu.cn